## BANDWIDTH AND DATA RATE

Bandwidth is a measure of how much data can be transmitted through a connection over a given period of time.  The common measurement for the Web is Kbps or Kilobits Per Second.

There are 8 bits in a byte so divide the bandwidth by 8 to figure out how many Kilobytes per second.

EXAMPLES

If you have a 5M file that is 20 seconds long then that will need a data rate of:

5 x 1000 x 8 / 20 = 2Mbps

If you have 20 second video and want it to run on a Modem (45 Kbps) then it would be compressed to:

45 / 8 * 20 = 112K

PERFORMANCE

If the data rate of a video is too high for the bandwidth then the video will stutter for streaming and progressive video (mov, rm, flv).  Of course bandwidth and file size (related to the data rate) will affect how long a video takes to be downloaded too if it is not streaming nor progressive.

SOUND

Remember that the data rate of your sound also needs to be considered.  So when you have video at 200Kbps and Audio at 96Kbps then you would need bandwidth of 296Kbps.  You might consider reducing the video to 256 – 96 = 160 Kbps to satisfy the 256K ISDN users.  See the chart below:

# Bandwidth Chart

| 28.8k Modem | 28.8 Kbps |
| --- | --- |
| 56K Modem | 56 Kbps (35-45) |
| ISDN 64K | 64 Kbps |
| ISDN 128K | 128 Kbps |
| ISDN 256K | 256 Kbps |
| ISDN 512K | 512 Kbps |
| Max 1.5M DSL | 1.235 Mbps |
| Max 3.0M DSL | 2.4 Mbps |
| T1 | 1.544 Mbps |
| T3 | 44.736 Mbps |
| OC-1 | 51.84 Mbps |
| OC-3 | 155.52 Mbps |

http://support.primus.ca/en/dsl/speedtest.php - to test your speed